1. Networks connect us

Communication is almost as important to us as our reliance on air, water, food and shelter

2. What is a network?

A network is a collection of two or more devices that are connected together to share resources, exchange data, and communicate with each other.

3. Host / End Device?

All devices that are connected to a network and participate directly in network communication are classified as hosts.

Hosts are also called end devices.

Example: Computers, Servers, Smartphones, Printers, and etc.

4. Client?

A client is a type of host that requests services from another device. Clients use software to request and display information received from servers.

5. Server?

A server is a type of host that provides services to other devices. Servers run software that allows them to provide information and services, often to multiple clients simultaneously.

Example:

When you open a website on your phone, your phone acts as a client. It sends a request over the network to a server where the website is hosted. The server receives this request, processes it, and sends back the required data, such as text, images, and videos. Your phone then displays this information on the screen through a web browser.

6. Intermediary devices

Network devices that connect hosts and help manage the flow of data across the network.

They do not generate or consume data like end devices, but instead forward, direct, and control data traffic between devices.

Examples:

- Routers

- Switches

- Wireless Access Points (WAPs)

- Firewalls

Intermediary devices:

- Forward data to the correct destination

- Determine the best path for data to travel

- Filter and secure traffic (Ex: firewalls)

- Connect different networks together

Important:

If hosts (end devices) are like people sending messages, intermediary devices are like the post office or traffic system that ensures the message reaches the correct destination efficiently.

7. Components of Data Communication System

- Sender:

The device that initiates (sends) the data.

- Receiver:

The device that receives the data.

Question:

Is the client always the sender and the server always the receiver?

No

A client and a server can both act as sender or receiver depending on the direction of communication. For example, when a client request data, it is the sender and the server is the receiver. When the server sends the requested data back, the server becomes the sender and the client becomes the receiver.

- Transmission medium:

The pathway through which the message travels from the sender to the receiver.

Guided Media (Wired): Data travels through a physical path such as cables.

Unguided Media (Wireless): Data travels through the air (no physical connection) using electromagnetic waves.

Question:

What is NIC?

NIC (Network Interface Card) is the hardware component that allows a device to connect to a network.

It acts as an interface between the device and the network, enabling the device to send and receive data.

NIC can be wired (Ethernet) or wireless (WNIC).

Modern computers usually have NICs built into the motherboard, Can add one if needed.

Every NIC has a unique identifier (MAC Address).

Institution of Electrical and Electronic Engineers

- Message:

The actual data or information that is being communicated between the sender and the receiver. It can be in a text, images, audio, video files, depending on the type of communication.

Question:

How data like text, images, or videos are send over a network?

All types of data whether text, images, audio, or video are represented and stored as binary (1s and 0s) inside a computer. When this data is transmitted over a network, the binary bits are encoded into physical signals. In wired networks, they travel as electrical or light signals, and in wireless networks, they travel as radio waves. Every message we send is a stream of binary data transmitted as signals.

Cables: Data is encoded into electrical impulses.

Fiber-Optic: Data is encoded into pulses of light.

Wireless transmission: Data is encoded via modulation of specific frequencies of electromagnetic waves.

Sending Data from Source to Destination

Data is encoded and transmitted as signals in the transmission media.

Based on the bit pattern of data, the sending side generates a signal on the selected transmission medium and then this signal propagates to the receiving end.

The receiving side extracts data from the signal and reproduce in its original format.

Question:

How signals are generated?

A signal is generated by changing the strength (or any other property) of a particular form of energy with respect to time on a suitable medium.

Form of energy used can be Electrical, Light or Electromagnetic.

Question:

What is Bandwidth?

Bandwidth is the maximum capacity of a channel.

Maximum amount of data that can be transmitted per second.

Question:

What is Throughput?

Throughput is the actual amount of data successfully transmitted per second.

Question:

Why Throughput < Bandwidth?

In real networks, we never achieve full bandwidth due to:

- Network congestion (too many devices)

- Packet loss & retransmissions

- Protocol overhead (headers, control data)

- Delays and interference

Actual data transfer (throughput) is always less than theoretical maximum (bandwidth).

Question:

In wireless networks, throughput is much lower. Why?

- Interference (walls, signals)

- Shared medium

- Signal fading

- Noise

Question:

What is Data Transmission Rate (Bit Rate)?

Number of bits send per second

Unit: bps

Question:

What is Baud Rate (Signaling Rate)?

The number of signal changes/symbols per second.

Unit: baud

Question:

Why Bit rate ≠ Baud rate (always)?

Because one signal (symbol) can represent more than one bit

Question:

Calculate Data Transmission Rate?

Given:

- Baud rate = 2400 symbols/sec

- Each symbol = 2 bits

Bit rate = Baud rate × bits per symbol

= 2400 × 2

= 4800 bps

- Protocol

Set of rules both sender and receiver must follow to ensure successful communication.

Types of Computer Networks

Computer networks can be classified according to their size.

- Personal Area Network (PAN)

- Local area network (LAN)

- Metropolitan area network (MAN)

- Wide area network (WAN)

Personal Area Network (PAN)

The smallest type of network.

Usually covers only a few meters.

Examples:

Connecting a mobile phone to wireless earbuds using Bluetooth.

Sharing internet from a phone to a laptop via a hotspot.

Local area network (LAN)

Covers a small geographic area such as a home, office, or school.

It connects devices like computers, printers, and servers within a limited space.

LANs provide high data transmission speed.

Usually privately owned and managed.

Enable the sharing of resources such as files or hardware devices that may be needed by multiple users.

Examples:

A computer lab in a school.

Office network.

Metropolitan area network (MAN)

Covers a larger geographic area such as a city or a large campus.

Formed by connecting multiple LANs together.

Example:

City-wide internet service.

Cable television network in a city.

Wide area network (WAN)

Covers a very large geographic area such as countries or continents.

Connects multiple LANs and MANs using long-distance communication links.

Have higher delays compared to smaller networks.

Example:

The internet

The Internet

The internet is a worldwide collection of interconnected networks.

Intranets and Extranets

An intranet is a private network that is used within an organization. It is accessible only to authorized users such as employees.

An extranet is an extension of an intranet that allows limited access to external users such as customers, suppliers, or business partners. It provides controlled access to certain parts of the organization’s network.

Creating a LAN

LAN’s can either be made wired or wireless.

Twisted pair, coax or fiber optic cable can be used in wired LAN’s.

Nodes are linked together with a certain topology.

Network topologies.

The physical or logical arrangement of devices (hosts) and how they are connected to each other in a network.

Defines how data flows and how devices communicate.

There are two main types of topologies.

Physical Topology and Logical Topology

Physical Topology:

The actual physical layout of cables and devices.

Shows how devices are physically connected.

Examples:

Bus, Star, Mesh, Ring, Tree, Hybrid

Star Topology

All devices are connected to a central device such as a switch or hub.

All communication passes through this central point.

Easy to manage and troubleshoot.

Failure of one device does not affect others.

If the central device fails, the whole network stops.

Bus Topology

All devices are connected to a single central cable called the backbone.

Data sent by one device travels along the cable and is received by all devices, but only the intended device processes it.

Simple and low-cost.

If the main cable fails, the entire network goes down.

Ring Topology

Devices are connected in a circular manner where each device is connected to two others.

Data travels in one direction around the ring until it reaches the destination.

Provides orderly data transmission.

Failure in one device or connection can disrupt the entire network.

Mesh Topology

Each device is connected to multiple other devices.

In a full mesh, each device is connected to every other device.

Provides high reliability and multiple paths for data.

Expensive and complex to set up due to the large number of connections.

Tree Topology

A hierarchical structure that combines characteristics of star and bus topologies.

Has a root node and branches out to multiple levels of devices.

Scalable and suitable for large networks.

Failure of higher-level devices can affect many lower-level nodes.

Hybrid Topology

A combination of two or more different topologies.

Example: star-bus, star-ring

Flexible and can be designed to meet specific network needs.

Can be complex and costly to implement and manage.

Logical Topology:

Refers to how data actually moves across the network regardless of the physical layout.

Question:

1. A network is physically arranged in a star topology but uses a hub. What is its logical topology?

The logical topology is Bus.

Because a hub broadcasts data to all connected devices. Similar to how data is transmitted in a bus topology.

2. A network uses a switch and is arranged in a star topology. What is its logical topology?

The logical topology is Star.

Because switch sends data only to the intended destination device.

3. Why is logical topology more important than physical topology in understanding network behavior?

Logical topology is more important because it defines how data flows between devices.

Network performance, efficiency, and troubleshooting depend on how data moves rather than how devices are physically connected.

Question:

Design a network for a school computer lab that consists of 10 student computers, 1 teacher computer, and 1 network printer. All devices should be connected using a switch in a star topology to form a Local Area Network (LAN). The network must allow all computers to communicate with each other and share the printer.

Packets

When data transfers through a network, it is not sent as a single large block. Instead, it is divided into smaller units called packets.

Packets are transmitted separately across the network and may take different paths. At the destination, they are reassembled in the correct order to form the original message.

This method is called packet switching.

Question:

Compare and contrast packet switching and circuit switching.

Packet switching and circuit switching are two fundamental methods of data transmission.

In circuit switching, a dedicated communication path is established between sender and receiver before any data is transmitted, and this path remains reserved for the entire session, even during idle periods

Example: Traditional telephone systems.

This results in fixed delay and data arriving in order, but it is inefficient because bandwidth is wasted when no data is being sent.

Packet switching breaks data into smaller packets.

Each packet travel independently across the network via different routes and be reassembled at the destination.

No dedicated path is reserved.

Network resources are used only when packets are actually transmitted.

Packet switching is more efficient and reliable.

Packet switching has variable delay. Packets may arrive out of order. Requires reassembly.

Packet switching is the foundation of the modern internet.

Question:

Each device in a network has a unique MAC address assigned to its NIC. Is the MAC address alone sufficient for communication in all types of networks?

MAC addresses alone are not sufficient for communication across multiple networks. Because MAC addresses do not provide any information about the location or network to which a device belongs.

Cannot use MAC address to determine the path for data to travel.

Solution: IP Addresses

IP Address

An IP address (Internet Protocol Address) is a unique numerical identifier assigned to each device connected to a network.

Use to uniquely identify a device on the network and to determine where the device is located in the network, so data can be sent to and from it.

Example:

IPV4 – 192.168.1.1

IPV6 - 2001:0db8:85a3:0000:0000:8a2e:0370:7334

Layered Network Architecture

Layering is a technique used to divide network communication into multiple layers.

Each layer performs a specific function.

Layered network architecture was introduced to overcome the limitations of proprietary systems.

Proprietary systems.

In the early days of computing, different companies such as IBM, Apple, and Microsoft developed their own communication systems. These were known as proprietary systems because they were designed to work only with that company’s own hardware and software.

In proprietary systems, each manufacturer used its own rules, formats, and protocols for communication. As a result, devices from different vendors could not easily communicate with each other.

Example:

IBM systems could communicate only with other IBM systems

Apple devices could not easily connect with Windows-based systems

Question:

How layered network architecture solves the problem of proprietary systems?

Layered network architecture solves the problem of proprietary systems by introducing standard rules at each layer of communication.

Instead of each company using its own complete system, all devices follow the same set of protocols at each layer.

This ensures that even devices from different manufacturers can communicate with each other, as they understand the same formats, addressing methods, and transmission rules.

OSI Model

The Open Systems Interconnection (OSI) model was developed by the International Organization for Standardization (ISO) to standardize communication between different computer systems.

OSI Model has 7 layers.

Application Layer:

Provides the interface for applications to access network services, such as email, file transfer, and web browsing.

Presentation Layer:

Handles data formatting, encryption, and compression to ensure compatibility between different systems.

Session Layer:

Manages communication sessions between applications, establishing, coordinating, and terminating connections.