Welcome everyone. Hope you are doing well. In this video we are going to discuss about OSI model. One of the most important concepts you need to understand well in networking. And it’s very interesting. We are going to fundamentally understand how devices communicate with each other.

Data communication is the exchange of data between two or more devices through a transmission medium such as a wire cable or wireless signal and data communication process involves 5 components.

Sender – The sender is the device that initiate communication. So he is the one who start transmitting the message. It could be a computer, smart phone, sensor or any digital device capable of sending data.

Receiver – Receiver is the device that accepts the message sent by the sender. It must be capable of interpreting and processing the received data. Like the sender this could be a computer, mobile device, or another electronic system.

Transmission Medium – Transmission medium is the pathway through which the message travels from the sender to the receiver.

It can be:

- Guided (wired): such as twisted pair cables, coaxial cables, or fiber-optic cables

- Unguided (wireless): such as radio waves, microwaves, or infrared

Then the Protocol – Protocol is simply the set of rules both sender and receiver must follow to ensure successful communication.

For example, when your browser (the sender) sends a request to a website (the receiver) using HTTP (HyperText Transfer Protocol), the website’s server must also understand and respond using the same protocol. If both ends follow HTTP correctly, the browser can successfully load the webpage. If not, the communication will fail.

It’s just like two people speaking different languages without understanding each other.

So the rules or main aspects in HTTP are

- Request and Response format : Every communication in HTTP follows a specific format.

A request from the client includes a method.

- GET: Retrieves data from the server.

- POST: Sends data to the server, often to create or update resources.

- PUT: Replaces a resource on the server with the data sent in the request.

- DELETE: Deletes a resource on the server.

These methods guide what client wants and how the server should respond. So the client makes a get request means

So the client makes a GET request, which means it is asking the server to send back some data like when you type a URL in your browser and press Enter, the browser sends a GET request to the server to fetch that page. Server process it and responds with the requested data, like an HTML file, and your browser displays it.

- Status Codes

When the server responds, it includes a status code to indicate the result of the request. Examples include:

- 200 OK: The request was successful

- 404 Not Found: The requested resource doesn't exist

- 500 Internal Server Error: A problem occurred on the server

These codes help the browser or client app understand what happened.

- Headers

Headers provide extra information about the request or response, such as:

- Content-Type: Tells what kind of data is being sent (e.g., HTML, JSON)

- User-Agent: Identifies the browser

- Authorization: Contains login credentials or tokens
Headers are key-value pairs that add context to the communication.

- Statelessness

Also HTTP is stateless, meaning each request is treated independently. The server does not remember anything about previous requests from the same client.

To maintain sessions (like staying logged into a website), additional tools like cookies or session management are used on top of HTTP.

As an example

So, if you log in to a website and move to another page, the server will forget you just logged in. Therefor the server verifies your credentials and sends back a cookie (e.g., session_id=12345) in the response. Then the browser saves the cookie. On every future request to that website, your browser automatically sends that cookie back to the server. The server reads the cookie, sees the session_id=12345, and says, "Ah! This is the same user who logged in." So you stay logged in across different pages.

- URL Structure

HTTP uses a Uniform Resource Locator (URL) to specify the location of the resource being requested.

https://www.example.com/products?page=2

This tells the server exactly which page or resource the client is trying to access.

So when communicating or sharing data over HTTP or HTTPS, the client and server (or sender and receiver) must follow these standard rules. These rules define how to send a request and how to interpret a response. So these are what we call a protocol.

Like HTTP we have different different protocols.

FTP (File Transfer Protocol)

- Used for transferring files between a client and server.

SMTP (Simple Mail Transfer Protocol)

- Used for sending emails from a client to a mail server.

TCP/IP (Transmission Control Protocol / Internet Protocol)

- The communication protocol of the internet.

- TCP ensures reliable delivery of data, IP handles addressing and routing.

DNS (Domain Name System)

- Translates domain names (like www.google.com) into IP addresses (142.250.190.4).

- Without DNS, we’d have to remember numeric IP addresses instead of website names.

Then the last component of Data communication, Message.

The message is the actual data or information that is being communicated between the sender and the receiver. It can be in a text, images, audio, video files, depending on the type of communication.

But here’s the thing you need to understand.

Computers are digital devices, which means they understand only binary — that is, 1s and 0s, which represent the presence or absence of electricity.

A = 01000001

So, all forms of data whether it's a message, a photo, or a video are first converted into binary before being transmitted. Each bit (binary digit) is sent as an electrical signal over the transition medium.

In wired communication (like Ethernet), this binary data travels through the cable as voltage pulses.
In wireless communication (like Wi-Fi or Bluetooth), it's sent as radio signals, but still represents 1s and 0s.

So, behind every image you send, every message you type, and every video you stream — it's really just a long stream of 1s and 0s traveling as signals, and that’s what we call the message in data communication.

Back in the early days of computing, companies like IBM, Apple, and Microsoft each had their own way of handling communication between devices. These systems were called proprietary systems. Which meaning they only worked with that company’s own hardware or software.

For example:

- IBM systems could only talk to other IBM systems.

- Apple devices couldn’t easily connect to Windows machines.

- Each manufacturer used its own rules and methods, making networking extremely limited and inefficient.

To solve this problem the OSI (Open Systems Interconnection) model was developed by ISO (International Organization for Standardization) in the late 1970s to:

The OSI model was developed to create a universal standard for communication. It allowed different hardware and software systems to talk to each other.

OSI Model has 7 layers. Think layer as a step in message sending process.

Because to send a message from one application to another device over a network, multiple tasks must happen in a specific order.

It’s like sending a letter. When sending a letter,

- You write the message

- You put it in an envelope

- You write the address and attach a stamp

- The postal service picks it up and delivers it

- Finally, the recipient opens and reads it

In the same way, when sending a message over a network:

- The message needs to be created

- The data needs to be formatted properly (so all devices understand it)

- A connection must be established between the sender and receiver

- The data must be split into smaller packets

- The best route to the destination must be found (using IP addresses)

- The correct receiving device must be identified (using MAC address)

- And finally, it must be converted into signals (electrical, optical, or radio) to travel through the medium

That’s why the OSI model divides communication into 7 layers, where each layer handles one specific task. This layered structure makes communication more organized, manageable, and allow devices from different manufacturers to communicate efficiently.

So the 7 layers of OSI model are,

Application
Presentation
Session
Transport
Network
Data Link
Physical

7. Application Layer:

Provides the interface for applications to access network services, such as email, file transfer, and web browsing.

6. Presentation Layer:

Handles data formatting, encryption, and compression to ensure compatibility between different systems.

5. Session Layer:

Manages communication sessions between applications, establishing, coordinating, and terminating connections.

4. Transport Layer:

Ensures reliable end-to-end delivery of data, including error detection and correction.

3. Network Layer:

Handles routing of data packets across networks, determining the best path for data to travel.

2. Data Link Layer:

Provides reliable transmission of data frames between two directly connected nodes.

1. Physical Layer:

Deals with the physical transmission of data over the network medium (e.g., cables, wireless signals).

Let’s take a real world example and understand this.

Imagine you are sending a message to your friend using WhatsApp on your phone while connected to Wi-Fi.

Layer 7: Application Layer

This is where you, the user, interact with the app.
You type "Hi!" and press Send. WhatsApp (the application) now prepares to send that data.

Layer 6: Presentation Layer
WhatsApp:

- Converts your message into a standard format (like text encoding),

- Compresses it to reduce data size,

- Encrypts it (end-to-end encryption) to ensure privacy.

Layer 5: Session Layer

A session (connection) is created between your phone and the WhatsApp server.
This ensures that as long as you’re chatting, the devices can send and receive continuously.

You can see that Layers 7 (Application), 6 (Presentation), and 5 (Session) are managed by the application software itself. That’s why in the TCP/IP model these three layers are combined into a single layer, and called the Application Layer.

Now let’s move on to the next layers.

Layer 4: Transport Layer

This layer is responsible for breaking the message into smaller chunks called segments and ensuring that they reach the destination reliably and in the correct order.

In our example:

- "Hi!" message into small segments.

- It uses a protocol like TCP (Transmission Control Protocol) to ensure no data is lost.

- If any segment gets lost or damaged, the Transport Layer requests it to be resent.

So, it’s like sending a letter in pieces, and making sure none of them go missing and they all arrive in the right order.

This breaking down of data into segments is done by the device’s operating system, specifically by its TCP/IP stack which is part of the network protocol stack built into your OS (like Windows, Linux, Android, iOS, etc.).

It’s always running in the background to manage your device's communication over the internet.

That defines how to:

- Break data into segments

- Label them with sequence numbers

- Send them reliably to the destination

- Reorder and check them at the receiver's side

- Ask for resends if anything is missing

So in transport layer our "Hi!" message gets broken into smaller parts and TCP assign sequence numbers.

| Segment | Sequence Number |
| --- | --- |
| Part 1 | 1001 |
| Part 2 | 1002 |
| Part 3 | 1003 |
| Part 4 | 1004 |
| Part 5 | 1005 |

At the receiving end, the device reassembles the message using the sequence numbers, even if the parts arrive out of order.

For example, if the server receives Part 3 before Part 2, it will wait until it gets Part 2 to arrange them properly before delivering to the app.

So what happen if a segment is lost or corrupted?

To that TCP includes error checking using something called a checksum. If the receiving side finds a segment is missing or corrupted, it can send a request (ACK) to the sender to resend that specific segment.

This is why we call TCP is a reliable protocol. It ensures:

- All data arrives

- In the right order

- Without corruption

Each TCP segment includes two main things:

Data: A portion of the original message (e.g., part of "Hi!" or larger messages)

Header: This is extra information used for tracking and managing the segments. Like:

- Sequence number: A number that shows what order this segment is in

- Acknowledgment number: Used to confirm which segments have been received

- Source and destination port numbers: Identify which app the data belongs to

- Flags: For connection setup, teardown, and control

- Checksum: For error checking

Once the data is broken into segments and prepared by the Transport Layer, it is passed down to the next layer.

So you can see Layer 4 (Transport) acts as a bridge between the application layers (Layers 5–7) and the network layers (Layers 1–3).

In the OSI model, Layers 5 to 7 (Session, Presentation, Application) are about:

- What the user wants to send

- How it's formatted

- And how the communication is managed

These are often handled by the application software like WhatsApp, a web browser, or Zoom.

On the other hand, Layers 1 to 3 (Physical, Data Link, Network) are about:

- How the data actually travels across networks (via cables, routers, Wi-Fi, etc.)

- And how to deliver that data from device to device

Layer 4 sits right in the middle:

It receives data from the application side (Layer 5–7):

For example, “Hi!” in WhatsApp and then prepares it for transport by breaking it into smaller chunks called segments and adding control information like Sequence number, Acknowledgment number, Ports, Error-checking info

Then it gives the prepared segments to the Network Layer (Layer 3).

Layer 3: Network Layer

This layer is responsible for routing the data across networks.

- It adds the source and destination IP addresses to each segment (now called a packet).

- It figures out the best path the data should take to reach the destination (e.g., WhatsApp server).

- This is the layer where routers work - forwarding data based on IP addresses.

Let’s understand this deeply. According to our example what was our network architecture?

Your mobile phone is connected to your home Wi-Fi router, and that router is connected to the internet. So, when you send a message, your phone sends data through the router, and the router handles communication with the internet.

Who gave us the router? That would be your ISP (Internet Service Provider).

When we purchase an internet connection from an ISP (like SLT, Dialog, or any other provider), they usually provide us a router, or you buy one separately and connect it to their service. This router is what we use to access the internet.

Alright, earlier we said that in the Network Layer, the system adds source and destination IP addresses to each segment, making it a packet.

The source IP address is your device’s IP.
In our WhatsApp example, this is the IP address of your mobile phone, which is the sender.

We know IP is a number.

IPV4 – 192.168.1.1

IPV6 - 2001:0db8:85a3:0000:0000:8a2e:0370:7334

This IP address is the unique identifier that allows devices to send and receive data over a network.

So who assign this IP to the Mobile Phone?

Well, when your phone connects to the Wi-Fi router, the router automatically assigns an IP address using a protocol called DHCP (Dynamic Host Configuration Protocol)

DHCP is a network management protocol used by routers and servers to automatically assign IP addresses and related configuration settings to devices on a network.

Your phone connects to the Wi-Fi -
It sends a request asking, “Can someone give me an IP address?”

The router responds -
“Yes, I’ll give you this one: 192.168.1.5 — and it’s yours for the next 24 hours.”

Your phone accepts it -
From that moment, your phone uses this private IP to communicate within the home network.

This makes life easier because we don’t have to manually type in IP addresses for each device.

In larger networks (like offices, or data centers), a dedicated DHCP server can be set up. This is done by installing DHCP server software on a computer or server — such as on Windows Server, Linux, or specialized network appliances.

Alright. But here is a twist.

The IP address your phone receives from the router is a private IP address. That means it’s only unique within your local network.

For example, your phone might get something like 192.168.1.5.

But you’re not the only person using 192.168.1.5. Millions of other people around the world might have devices with that exact same IP inside their own home networks.

So you can understand private IP addresses are reused. They can’t be used to communicate directly with the internet.

Then, how can the WhatsApp server send a response back to you?

This gets handled by the router by doing something called NAT (Network Address Translation).

Your router is assigned a public IP address by your ISP. This is globally unique and visible to the internet (for example: 112.134.5.25).

When your phone (with private IP 192.168.1.5) sends a message (data packet) to the WhatsApp server, the router:

- Replaces the private IP with its own public IP.

- Keeps a record (in the NAT table) that says:
“Hey, this request came from 192.168.1.5 — I’ll remember that.”

- Sends the packet to WhatsApp using its public IP.

When WhatsApp responds, the reply goes to the router’s public IP address. Because that’s the IP used in the request.

Then the router:

- Checks its NAT table to see which internal device requested it.

- Forwards the response to the correct private IP. In this case, your phone at 192.168.1.5.

Ok that’s about source IP address. What about destination IP. How can the mobile phone know WhatsApp servers public IP address?

Actually your phone doesn’t know the IP address of WhatsApp’s server by default. It only knows the domain name, like,

web.whatsapp.com
api.whatsapp.com

So it needs to translate that domain name into an IP address. That’s done by the DNS (Domain Name System)

DNS is like the phonebook of the internet. It takes human-readable names (like whatsapp.com) and resolves them into machine-readable IP addresses (like 157.240.14.60).

To find the source IP address,

1. Your phone checks its local DNS cache
If you’ve recently contacted WhatsApp, your device might already remember the IP.

2. If not, the phone sends a DNS request (usually to the DNS server provided by your ISP or Google DNS like 8.8.8.8).

3. The DNS server looks up the domain (api.whatsapp.com) and responds with the IP address of the server — for example, 157.240.14.60.

Now your phone knows the destination IP, and adds it to the packet.

At the Network Layer (Layer 3), the system adds several other important fields to the packet — forming what’s called the IP header.

IP Header Fields (at Layer 3 – Network Layer):

- Source IP Address
– The IP address of the sender (your device)

- Destination IP Address
– The IP address of the receiver (e.g., WhatsApp server)

- Version
– Indicates the IP version: IPv4 or IPv6

- Header Length
– Specifies how long the header is (since it can vary in size)

- Total Length
– The full size of the packet, including both the header and the actual data (payload)

- TTL (Time To Live)
– Limits how many hops (routers) the packet can go through before it’s discarded
(Each router decreases this value by 1 — helps avoid endless looping)

- Protocol
– Tells what kind of data is inside the packet (e.g., TCP = 6, UDP = 17)

- Identification, Flags, and Fragment Offset
– Used when the packet is too large and needs to be split (fragmented) across multiple transmissions

- Header Checksum
– A value used to detect errors in the header during transmission

And you can remember at layer 4 – Transport layer the source and destination port numbers were added to the segment.

So, what do we mean by adding source and destination port numbers?

Well, imagine you’re sending data from your WhatsApp app. Your phone could be running many apps at the same time — like YouTube, Gmail, and WhatsApp — and they all use the same IP address. So how does the system know which app is sending or receiving the data?

That’s why we need port numbers.

A port is simply a number used to identify a specific application or service on a device. In the Transport Layer, when the message is broken into segments, each segment

Gets a source port number (the app sending the data — e.g., WhatsApp on your phone)

And a destination port number (the app that should receive the data — e.g., WhatsApp’s server).

These numbers are added into the TCP segment header. Then your device and the server know exactly where to send the data internally.

For example, port 443 is commonly used for secure HTTPS connections.

It’s like you're in a large apartment building (your device), and each apartment (port) belongs to a different person (app). The building has one address (your IP), but many apartment numbers (port numbers). If someone sends a letter to that building, just writing the building address isn't enough — they also need to write the apartment number so the letter goes to the right person.

Alright then from Layer 3 – Network layer this data packet will get passed to Layer 2 – Data Link Layer.

Layer 2: Data Link Layer

At this layer, the main responsibility is to prepare the packet for travel across the local network.

It takes the packet received from the Network Layer (Layer 3) and wraps it inside a new structure called a frame.

A frame is like a local delivery package. It includes:

- The original packet (from Layer 3)

- A header that contains:

  - Source MAC address (e.g., your mobile phone’s network card)

  - Destination MAC address (e.g., your router’s MAC address)

- A trailer that often includes:

  - Error-checking data (like CRC – Cyclic Redundancy Check)

So what is this MAC address?

A MAC address (Media Access Control address) is a unique hardware address given to every network device (like your phone, laptop, or router) (NIC) by the manufacturer.
It’s used to identify devices within a local network (LAN).

A MAC address looks like this:
00:1A:2B:3C:4D:5E (hexadecimal format – 6 pairs of characters)

We often refer to the MAC address as a Burned-in Address, meaning it’s hard-coded into the hardware and usually does not change.

Then you might ask — if every device already has a unique MAC address, why do we even need an IP address, especially a public IP address?

Because purpose of these two addresses are different.

A MAC address is only used for local communication within the same Local Area Network (LAN). An IP address especially a public IP use to communicate over the internet.

As an example

Imagine you're sending a birthday card to your friend Alex, who lives in a large apartment building.

- The house/apartment address is:
No. 24, Green Lane, Colombo
This is like the IP address — it tells the postal service where to deliver the letter (the location on the network).

- But inside that apartment, there are many people — Alex, his brother, his parents...
So on the envelope, you also write:
To: Chamindu
That’s like the MAC address — it tells the building which specific person should receive the letter.

So we can say MAC addresses are hardware addresses. They're unique to your device, but they don't say where the device is located on the network. Or we can say MAC addresses are used for device-to-device delivery, while IP addresses are used when we need to move data across networks.

You can remember in our previous video we discuss about switch. Switch uses these MAC addresses to forward the frame to the correct device.

When a device sends a frame, the switch sees the source MAC and remembers which port it came from. It stores that in a table called the MAC Address Table (also called a CAM table).

| MAC Address | Port |
| --- | --- |
| 00:1A:2B:3C:4D:5E | Port 1 |
| 11:22:33:44:55:66 | Port 2 |

Next time it receives a frame for 11:22:33:44:55:66, it knows exactly which port to send it to.

So What If the Switch Doesn’t Know the MAC Yet?

It floods the frame to all ports except the one it came from. Once the correct device replies, the switch learns where that MAC lives.

Without this MAC addresses and switching every message would have to be broadcasted to every device (very inefficient), there would be data collisions, slower speeds.

So according to our WhatsApp example,

When you type "Hi!" and press send, WhatsApp handles it at the Application, Presentation, and Session layers by formatting, compressing, and encrypting the message while establishing a session with the server.

Then at the Transport Layer (Layer 4), the message is broken into smaller segments using TCP, which assigns sequence numbers and adds both the source and destination port numbers.

These segments are then passed to the Network Layer (Layer 3), where they are wrapped into packets. At this point, the source IP address (e.g., 192.168.1.5, your phone’s private IP) and destination IP address (e.g., 157.240.14.60, WhatsApp’s public server IP) added.

But your phone cannot directly reach 157.240.14.60 — it’s outside your local network. So the phone must send the packet to your router (default gateway) at 192.168.1.1.

The router then performs NAT (Network Address Translation) — this means it replaces the source private IP address (192.168.1.5) with its own public IP address (assigned by your ISP) before forwarding the packet to the internet.

But now you can see there is a problem. To send the packet to router phone need to know routers MAC address?

To find routers MAC address (IP address - 192.168.1.1) your phone uses a protocol called ARP (Address Resolution Protocol).

- The phone sends an ARP broadcast message on the local network asking:
“Who has IP address 192.168.1.1? Please send me your MAC address.”

The phone knows the router’s IP address because it is configured as the default gateway for the device.

When your phone connects to the Wi-Fi network, it automatically receives several pieces of network information from the router (or another DHCP server)

The phone’s private IP address (e.g., 192.168.1.5)

The subnet mask (which defines the local network range)

The default gateway IP address (usually the router’s IP, e.g., 192.168.1.1)

DNS server addresses

The default gateway IP is the address the phone uses to send packets destined for outside its local network. So, whenever the phone needs to send data to an IP address that isn’t in its own subnet, it sends the packet to this default gateway

Then when after this ARP request send

- The router recognizes its IP and replies with its MAC address.

- The phone then caches this MAC address and uses it to send the frame containing the packet directly to the router.

This MAC address is used only inside your local network — outside the local network, IP routing and NAT take over to forward the data across the internet.

So the flow in the local network looks like this:
Phone (source MAC, source IP) → Router (destination MAC, router IP)

Once the router gets the frame, it strips off the Layer 2 information, performs NAT (Network Address Translation). NAT replaces the source IP (your phone’s private IP, e.g., 192.168.1.5) with the router’s public IP address (e.g., 112.134.88.10), which was assigned by your ISP.

After performing NAT, the router encapsulates the packet into a new frame, this time using its own MAC address as the source and the MAC address of the next hop (usually your ISP’s router) as the destination.

Then, the data is passed to Layer 1 (Physical Layer), where it’s converted into electrical signals (over Ethernet), radio signals (for fiber or wireless), or light pulses (for fiber optics), and sent out over the internet.

  The frame reaches the next hop — typically your ISP’s router.

  At each router along the way:

- The Layer 2 frame is stripped off, and a new frame is created using that router’s MAC address and the MAC of the next hop.

- The Layer 3 IP packet stays the same, but TTL (Time To Live) is decreased by 1.

- This repeats hop-by-hop, across various routers over the internet.

This process continues until the packet arrives at the WhatsApp server’s network.

When the packet finally reaches the data center where the WhatsApp server is located:

- The server’s router receives the final Layer 2 frame.

  - It strips off the frame.

  - Sees the destination IP matches one of its internal servers (e.g., 157.240.14.60).

  - Forwards the packet to the WhatsApp server machine.

- At the WhatsApp server:

  - The IP packet is passed up to Layer 4 (Transport Layer).

  - The server sees:

    - Source IP: Your router’s public IP (after NAT).

    - Destination IP: The server’s IP.

    - Source Port: Identifies the user's session on your device.

    - Destination Port: (e.g., port 443 for HTTPS communication).

- The server’s TCP protocol uses the sequence numbers in the segment to:

  - Reassemble multiple segments into the full message (if needed).

  - Check for errors (using checksums).

  - Send acknowledgments for each received segment.

- Once all the data is received and verified:

  - The Session Layer maintains the session between you and the app.

  - The Presentation Layer decrypts and decompresses the message.

  - The Application Layer finally delivers the "Hi!" to the correct WhatsApp contact on the server side.

### 📲 Then What?

- The WhatsApp server processes the message and decides where it should go next — e.g., to your friend's phone.

- It repeats the same OSI model process in reverse to send a response back — which again travels over the internet, reaches your router, and finally gets delivered to your phone.